DAYANANDA SAGAR COLLEGE OF

ARTS, SCIENCE AND COMMERCE

BANGALORE – 560111

Master of Computer Applications (MCA)

Project On :

"4-Bit Binary Calculator Using 1's and 2's Complement"

Submitted by:

Kirti Hosamath – P03CJ25S126028

Bijaylaxmi Nayak – P03CJ25S126013

Chinmayi M D – P03CJ25S126016

Submitted to:

Prof. Akshata Deepak

Department of Computer Applications

TABLE OF CONTENTS

| Sl. No. | Section | Page No. |
| --- | --- | --- |
| 1. | Introduction and Objective | 03 |
| 2. | Block Diagram | 04 |
| 3. | Simulator Details and Implementation Files | 06 |
| 4. | Implementation | 08 |
| 5. | Test Cases and Results | 10 |
| 6. | Conclusion | 12 |
| 7. | Link of Working Project | 14 |

# 1. INTRODUCTION AND OBJECTIVE

## A. Introduction

The 4-Bit Binary Calculator is a digital computing project implemented in Python using the Tkinter GUI framework. It simulates fundamental binary arithmetic operations, demonstrating how a computer processes numeric data at the bit level. The calculator supports four core operations: binary addition, binary subtraction (via 2's complement), 1's complement, and 2's complement computation.

This project provides a hands-on simulation of bit-level computation through a graphical desktop application complete with an LED display panel, input fields, operation buttons, and real-time carry/overflow detection. The program reflects the foundational logic used in Arithmetic Logic Units (ALUs) of modern processors.

## B. Objective

The primary objectives of this project are:

- Arithmetic Simulation: To design and simulate a 4-bit binary calculator capable of performing addition and subtraction at the bit level.

- Complement Logic: To implement 1's complement (bitwise inversion) and 2's complement (negation) operations to enable subtraction through addition logic.

- Error Detection: To detect and display carry-out and arithmetic overflow, distinguishing between unsigned and signed arithmetic errors.

- GUI Visualization: To build an interactive graphical user interface (GUI) using Python's Tkinter library, including an LED display panel, keypad inputs, and operation controls.

- Educational Purpose: To demonstrate how binary subtraction is achieved using the 2's complement method — the same technique used in real hardware ALUs.

- Result Export: To allow users to export results to a text file for documentation and further analysis.

# 2. BLOCK DIAGRAM

The 4-Bit Binary Calculator follows a structured data flow from input to output. The block diagram below illustrates the overall architecture of the system, including the input layer, complement logic, arithmetic unit, and output/display layer.

## A. System Architecture Overview

The calculator processes two 4-bit binary numbers (A and B) through the following stages:

- Input Layer: User enters binary numbers via the keypad or text fields (Input A and Input B).

- Complement Logic Unit: Depending on the operation selected, B may be transformed to its 1's complement (bitwise NOT) and then incremented by 1 to produce the 2's complement.

- Arithmetic Unit: The binary adder performs ripple-carry addition of A and the (possibly transformed) B, tracking each bit-level carry.

- Output and Flags: The final 4-bit result is displayed, along with the carry-out and overflow flags.

- LED Display: The result bits illuminate corresponding LED indicators (S3, S2, S1, S0), with a separate Carry LED.

## B. Binary Addition Flow

For addition (A + B), both inputs are normalized to 4 bits and fed directly into the ripple-carry adder. Carry propagates from bit 0 (LSB) to bit 3 (MSB). If a carry exits beyond bit 3, the carry-out LED turns red.

## C. Subtraction via 2's Complement

For subtraction (A - B), the system computes the 2's complement of B as follows:

Step 1: 1's Complement of B  =  bitwise NOT (B)

Step 2: 2's Complement of B  =  1's Complement(B) + 0001

Step 3: Result               =  A + 2's Complement(B)

This approach mirrors the hardware behaviour of a real Adder-Subtractor circuit, where a mode bit (M=1) triggers XOR-based inversion of B inputs and injects an initial carry of 1 into the adder chain.

## D. Overflow Detection

Overflow is detected by comparing the carry into the most significant bit (C3) and the carry out of it (C4). If they differ, the signed result is invalid:

overflow = C3 XOR C4

## E. LED Output Panel

| LED | Label | Color | Indicates |
| --- | --- | --- | --- |
| Carry LED | Carry | Red | Carry-out from MSB |
| LED 0 (S3) | MSB | Lime Green | Bit 3 of result |
| LED 1 (S2) | Bit 2 | Lime Green | Bit 2 of result |
| LED 2 (S1) | Bit 1 | Lime Green | Bit 1 of result |
| LED 3 (S0) | LSB | Lime Green | Bit 0 of result |

# 3. SIMULATOR DETAILS AND IMPLEMENTATION FILES

The project is implemented as a single Python desktop application using the Tkinter library. The file with_carry.py serves as the complete implementation — containing both the computational logic and the graphical user interface.

## A. Implementation File

| File | Purpose | Technology |
| --- | --- | --- |
| Binary_calculator.py | Full application — logic + GUI | Python 3 + Tkinter |

## B. GUI Features

The graphical interface is built with Tkinter and provides the following features:

- Input Fields: Two text entry fields for 4-bit binary Input A and Input B, each editable via keyboard or the on-screen keypad (0 and 1 buttons).

- Operation: ADD: ADD — performs 4-bit binary addition with carry detection.

- Operation: SUB: SUB — computes subtraction using 2's complement addition.

- Operation: 1C: 1's Complement — computes the bitwise inverse of Input A.

- Operation: 2C: 2's Complement — computes the negation of Input A.

- Operation: Clear: Clear — resets all fields and LEDs to their default state.

- Export: Export Result — saves the current inputs and result to a user-specified .txt file.

## C. LED Display Panel

A Tkinter Canvas widget renders five circular LED indicators at the bottom of the window. Four green LEDs (S3–S0) represent the 4-bit binary result. A red Carry LED lights up when a carry-out is produced during addition. The LEDs update in real time after every operation.

## D. Overflow Indicator

A yellow label below the result field displays the Overflow status (0 or 1) after each operation. Overflow detection is active only for addition, as it requires comparing the carry into and out of the most significant bit.

# 4. IMPLEMENTATION

The implementation details the Python logic used to simulate 4-bit binary arithmetic. All arithmetic is performed at the bit level without relying on Python's built-in integer arithmetic for the core computation.

## A. Input Normalization

Both inputs are zero-padded to 4 bits before any operation:

def normalize_4bit(a, b):

return a.zfill(4), b.zfill(4)

## B. Binary Addition with Carry

The adder iterates from bit 3 (LSB) to bit 0 (MSB), summing corresponding bits and tracking carry at each stage:

def binary_addition(a, b):

result = ''

carry = 0

carries = []

for i in range(3, -1, -1):

total = int(a[i]) + int(b[i]) + carry

result = str(total % 2) + result

carry = total // 2

carries.insert(0, carry)

overflow = carries[0] ^ carry

return result, carry, overflow

## C. 1's Complement

Each bit is individually inverted:

def ones_complement(binary):

return ''.join('1' if bit == '0' else '0' for bit in binary)

## D. 2's Complement

Computed by taking the 1's complement and adding 0001:

def twos_complement(binary):

ones = ones_complement(binary)

result, _, _ = binary_addition(ones, '0001')

return result

## E. Binary Subtraction

Subtraction is performed by adding the 2's complement of B to A:

def binary_subtraction(a, b):

b_twos = twos_complement(b)

result, carry, overflow = binary_addition(a, b_twos)

return result, overflow

## F. Full Adder Truth Table

| A | B' | Cin | Sum (S) | Carry Out |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

# 5. TEST CASES AND RESULTS

The calculator was verified using the following test cases. All operations were validated manually and confirmed against expected results.

## A. Addition Test Cases

| Input A | Input B | Result | Carry | Overflow | Status |
| --- | --- | --- | --- | --- | --- |
| 0011 (3) | 0010 (2) | 0101 (5) | 0 | 0 | Correct |
| 0111 (7) | 0001 (1) | 1000 | 0 | 1 | Overflow Detected |
| 1111 (15) | 0001 (1) | 0000 | 1 | 0 | Carry Out |
| 0000 (0) | 0000 (0) | 0000 (0) | 0 | 0 | Correct |

## B. Subtraction Test Cases

| Input A | Input B | Result | Decimal | Overflow | Status |
| --- | --- | --- | --- | --- | --- |
| 0111 (7) | 0011 (3) | 0100 | 4 | 0 | Correct |
| 0011 (3) | 0101 (5) | 1110 | -2 | 0 | Correct (Borrow) |
| 0101 (5) | 0101 (5) | 0000 | 0 | 0 | Correct (Zero) |

## C. Complement Test Cases

| Input A | Operation | Result | Status |
| --- | --- | --- | --- |
| 0101 (5) | 1's Complement | 1010 | Correct |
| 0101 (5) | 2's Complement | 1011 | Correct |
| 1111 | 1's Complement | 0000 | Correct |
| 0000 | 2's Complement | 0000 | Correct (Zero) |

## D. Summary of Results

- Accuracy: All binary addition operations produce correct 4-bit results, with accurate carry-out detection.

- Subtraction: Subtraction via 2's complement correctly computes differences including negative results in signed representation.

- Overflow: The overflow flag correctly triggers only when the signed result exceeds the 4-bit range [-8, +7].

- Complement: 1's and 2's complement functions produce correct outputs for all 4-bit patterns including edge cases (0000, 1111).

- LED Display: The LED panel and carry indicator update correctly in real time after each operation.

# 6. CONCLUSION

## A. Summary and Key Findings

The 4-Bit Binary Calculator project successfully demonstrates how fundamental binary arithmetic can be implemented at the bit level in software. Using Python and Tkinter, the project simulates the core operations of a hardware Arithmetic Logic Unit (ALU) — including ripple-carry addition, 2's complement subtraction, and complement generation — in an interactive desktop application.

Key technical insights demonstrated by this project include:

- Hardware Efficiency: Subtraction does not require a separate hardware circuit. By computing the 2's complement of the subtrahend and adding it to the minuend, the same adder logic serves both addition and subtraction, mirroring real ALU design.

- Data Representation: Binary data is inherently neutral. Its interpretation as signed or unsigned depends on context — the same 4-bit pattern can represent both a positive unsigned integer and a negative signed value via 2's complement representation.

- Error Diagnostics: Carry-out and Overflow flags serve distinct diagnostic purposes. Carry-out indicates unsigned range overflow; the XOR of the carry into and out of the MSB detects signed overflow.

- Visualization: GUI simulation with LED feedback provides an intuitive educational tool for understanding bit-level operations, making abstract concepts tangible.

## B. Learning Outcomes

Through this project, the following competencies were developed and demonstrated:

- Binary Logic: Bit-level manipulation in Python using string operations and Boolean logic.

- Arithmetic Design: Implementation of ripple-carry addition and carry propagation.

- Complement Arithmetic: Application of 1's complement inversion and 2's complement negation for signed arithmetic.

- GUI Development: Building interactive desktop GUIs using Python's Tkinter framework including Canvas-based LED rendering.

- Error Detection: Implementing overflow detection using XOR logic on carry signals.

## C. Limitations

- Bit-Width Constraint: The 4-bit architecture restricts the numeric range to 0–15 (unsigned) or −8 to +7 (signed), limiting practical utility.

- Limited Operations: Only addition, subtraction, and complement operations are supported; multiplication, division, and logical operations (AND, OR, XOR) are not included.

- GUI Simplicity: The GUI uses a basic Tkinter design; it lacks responsive layout or touchscreen support.

- Input Handling: No input validation beyond length checking — non-binary characters in the input fields can cause runtime errors.

## D. Future Scope

- N-Bit Expansion: Expand to 8-bit or 16-bit arithmetic to support a wider numeric range and practical computation.

- Full ALU: Add logical operations: AND, OR, XOR, NOT, NOR, and NAND to create a full ALU simulator.

- CLA Optimization: Implement a Carry Lookahead Adder (CLA) to reduce sequential carry delay compared to the current ripple-carry approach.

- Step-by-Step Trace: Add a step-by-step trace panel showing intermediate carries, complement values, and bit-level operations for educational use.

- Hardware Prototype: Implement physical prototyping on an FPGA or breadboard using 74-series logic ICs to validate the software-level logic in hardware.

# 7. LINK OF WORKING PROJECT

The complete source code for this project is available in the following file:

| File | Description |
| --- | --- |
| Binary_calculator.py | Main Python application — includes all logic and GUI code |

To run the project:

# Requirements: Python 3.x with Tkinter (standard library)

python with_carry.py

## Project Repository

The source code and supporting files are hosted at:

GitHub Repository: https://github.com/[your-username]/4bit-binary-calculator

## How to Run

- Step 1: Ensure Python 3.x is installed on your system (Tkinter is included by default).

- Step 2: Download or clone the repository.

- Step 3: Navigate to the project directory in your terminal.

- Step 4: Run the command: python with_carry.py

- Step 5: The GUI window will open. Enter 4-bit binary values in Input A and Input B and select an operation.